Personal Face Recognition System Using MTCNN and DeepFace (FaceNet)

## Introduction

In this project, a face recognition system has been created using Python in Google Colab to identify and classify whether a face image uploaded matches the user or not. A small face image database with four face images was generated; two of them are the user's face image, and the other two are images of other people. The objective of the system is to find the best match by comparing an uploaded image with the images in the database. This implementation integrates face detection and deep learning-based face verification methods for precise face recognition.

## Methodology

The preparation of a database has been carried out using four facial images for the recognition process. The MTCNN (Multi-task Cascaded Convolutional Networks) algorithm was used to locate the faces in images of each of the two databases before comparing images. The MTCNN determines the region of the face through face detection and a surrounding bounding box, and only valid regions of the face will be fed into the recognition process. Firstly, face detection was performed, and the DeepFace library was employed with the FaceNet pre-trained model for face verification. Individual comparisons between the uploaded image and each image stored in the database were made. FaceNet creates deep embeddings for faces in images and computes the Euclidean distance between them. Lower distance values mean higher similarity, and higher distance values mean that they are different identities.

## Results

In each comparison, the system returned the verification result (True/False) and the similarity distance that was calculated. The data obtained after the comparison was stored and analysed to find out the verified matches. If at least one match was verified, then the system picked out the image with the smallest Euclidean distance as the best match and showed the identity of the selected image. When no matching faces were found, the system correctly said "Unknown Person" and "This is NOT your face”. This behaviour shows that the recognition system is capable of identifying known and unknown persons, unlike a forced match.

## Conclusion

The face recognition system implemented is successfully integrated with face detection using MTCNN and face verification using the DeepFace and FaceNet deep learning models. The system identifies the facial region, makes a comparison between the image input to the system and the image stored in the system, obtained by calculating the Euclidean distance, and determines the best match. The final implementation fulfils the coursework objectives by enabling the uploading of face images, comparison of similarity, face verification, as well as identification of personal faces using deep learning techniques.